CMPG 325 — COMPUTER NETWORKS

INDIVIDUAL PROJECT · MILESTONE 1

Client Design Review — Tshimo Agri Supplies

| Student | Maluleke, Kurhula Success |
| --- | --- |
| Student Number | 48277444 |
| Project ID | CMPG325-2026-042 |
| Client ID | CLI-042 |
| Assigned Organisation | Tshimo Agri Supplies (Potchefstroom) |
| Industry | Agriculture |
| Assigned Addressing Block | 10.23.0.0/16 |
| Assigned Networking Challenge | Network Troubleshooting (fault isolation scenario) — Advanced |

# Contents

1.   Client Requirements

2.   Physical Topology

3.   Logical Topology

4.   IP Addressing Plan

5.   Initial GitHub Repository

Design Philosophy Every decision in this plan follows one rule: solve the requirement by allocation, not by re-architecture. Uniform subnet sizes, a single summarizable HQ route, and untouched reserved space mean the branch-office constraint and CR1 are both already solved before they happen.

# 1.  Client Requirements

## 1.1  Stated Requirements (from project brief)

- Assigned organisation: Tshimo Agri Supplies (Potchefstroom); Industry: Agriculture.

- Assigned addressing block: 10.23.0.0/16 — must form the basis of the entire IP addressing plan.

- Provide appropriate connectivity and network services for the assigned scenario.

- Design Constraint: a branch office may be opened within 18 months — the addressing plan must allow for it without redesign.

- Client Change Request (CR1): the client signs 8 additional staff in one department — capacity must be added without redesigning the network.

- Produce a working, testable Cisco Packet Tracer implementation, and demonstrate the assigned challenge: Network Troubleshooting (fault isolation scenario).

## 1.2  Interpreted Organisational Requirements (design assumption)

The brief does not specify Tshimo Agri Supplies' internal department structure, so the following breakdown was adopted for a small-to-medium agricultural supply business. This assumption drives the topology and addressing plan below.

| Department | Function | Current Staff |
| --- | --- | --- |
| Administration / Management | HR, finance, general management | 8 |
| Sales | Customer-facing sales & orders | 12 |
| Warehouse / Logistics | Stock, dispatch, delivery (CR1 department) | 12 (+8 via CR1 = 20) |
| IT / Server Room | DHCP, DNS and File/ERP services | 3 servers |

CR1 is applied to the Warehouse / Logistics department, since staffing growth in an agricultural supply business most plausibly occurs in stock handling and dispatch as client volume grows.

# 2.  Physical Topology

The physical topology shows the devices, device roles and cabling for the Tshimo Agri Supplies head office in Potchefstroom. A single edge router provides the WAN connection; a core switch distributes 802.1Q trunks down to one access switch per department, keeping every department electrically and logically separate while sharing a single router for inter-department routing.

Figure 1 — Physical Topology, Tshimo Agri Supplies HQ (Potchefstroom)

| Device | Role | Suggested PT Model | Key Interfaces |
| --- | --- | --- | --- |
| R1 | Edge router — WAN uplink, inter-VLAN routing (router-on-a-stick), NAT/PAT | Cisco 4321 / 2911 | Gi0/0 (trunk to SW-CORE), Gi0/1 (WAN to ISP) |
| SW-CORE | Distribution switch — trunks VLANs between R1 and access layer | Cisco 2960 | Trunk ports to R1 and each access switch |
| SW-ADM / SW-SAL / SW-WH | Access switches — one per department | Cisco 2960 (24-port) | Access ports to PCs, uplink trunk to core |
| SW-SRV | Access switch — server segment | Cisco 2960 | Access ports to DHCP/DNS/File servers |
| End devices | Department PCs + 3 servers | PC-PT / Server-PT | FastEthernet to access switch |

Note: access switches are specified as 24-port models even where current headcount is lower (e.g. Admin at 8 PCs), so CR1-style headcount growth is absorbed at the physical layer as well as the addressing layer — see Section 4.

# 3.  Logical Topology

Logically, each department is a separate VLAN. R1 performs inter-VLAN routing using a router-on-a-stick design (one sub-interface per VLAN on Gi0/0), and provides the default route out to the ISP on Gi0/1 with NAT/PAT for internet-bound traffic. DHCP is served from a dedicated DHCP server in the Server VLAN rather than the router, to reflect realistic enterprise practice.

Figure 2 — Logical Topology: VLANs, subnets and default gateways

| VLAN | Name | Purpose | Subnet |
| --- | --- | --- | --- |
| 10 | ADMIN | Administration / Management | 10.23.0.0/26 |
| 20 | SALES | Sales | 10.23.0.64/26 |
| 30 | WAREHOUSE | Warehouse / Logistics (CR1 department) | 10.23.0.128/26 |
| 40 | SERVERS | DHCP, DNS, File/ERP servers | 10.23.0.192/26 |
| 99 | MGMT-NATIVE | Switch management / native VLAN (no user traffic) | 10.23.1.16/28 |

# 4.  IP Addressing Plan

The assigned block 10.23.0.0/16 is split first at the /17 level: the lower half is allocated to the Potchefstroom HQ site now, and the upper half is reserved, untouched, for the branch office constraint. Inside HQ, the plan uses exactly two /24 networks today — one for business VLANs, one for infrastructure — chosen so the whole HQ site collapses into a single summary route upstream.

Why this is the smart solution, not the hard one 1. Uniform block sizing — every department VLAN is the same size (/26 = 62 hosts), so the whole plan fits in one sentence: 'VLAN N lives at offset (N/10 − 1) × 64 in 10.23.0.0/24.' Nothing to memorise per-department. 2. One summary route — 10.23.0.0/24 (business) and 10.23.1.0/24 (infrastructure) are contiguous, so R1 can summarise the entire HQ site as a single 10.23.0.0/23 route upstream instead of advertising every subnet individually. 3. Separation of concerns — user traffic (business VLANs) and infrastructure (WAN, management, guest) live in different /24s, a real enterprise security practice, not just a numbering trick. 4. Growth by allocation, not redesign — CR1 and the branch-office constraint are both solved by handing out already-reserved space, never by re-cabling, re-VLANing or re-IP-addressing anything already deployed.

## 4.1  Top-level split — Design Constraint (branch office)

| Allocation | Block | Status |
| --- | --- | --- |
| HQ — Potchefstroom | 10.23.0.0/17 | In use — 2 of 128 possible /24s (see 4.2 / 4.3) |
| Reserved — future Branch Office | 10.23.128.0/17 | Untouched — available on demand |

## 4.2  Business VLANs — 10.23.0.0/24 (uniform /26 blocks)

| Subnet (VLAN) | Network / CIDR | Usable Range | Broadcast | Gateway | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| VLAN 10 — Admin | 10.23.0.0/26 | .1 – .62 | .63 | .1 | 62 (8 used) |
| VLAN 20 — Sales | 10.23.0.64/26 | .65 – .126 | .127 | .65 | 62 (12 used) |
| VLAN 30 — Warehouse | 10.23.0.128/26 | .129 – .190 | .191 | .129 | 62 (20 used) |
| VLAN 40 — Servers | 10.23.0.192/26 | .193 – .254 | .255 | .193 | 62 (3 used) |

## 4.3  Infrastructure — 10.23.1.0/24 (kept separate from user VLANs)

| Purpose | Network / CIDR | Usable Range | Broadcast | Gateway | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| R1 – ISP WAN link | 10.23.1.0/30 | .1 – .2 | .3 | n/a | 2 (point-to-point) |
| VLAN 99 Mgmt/Native | 10.23.1.16/28 | .17 – .30 | .31 | .17 | 14 (switches only) |
| Guest Wi-Fi (future) | 10.23.1.32/28 | .33 – .46 | .47 | .33 | 14 (reserved) |
| Free / future infra | 10.23.1.48 – .255 | — | — | — | 208 addresses spare |

## 4.4  How this plan satisfies the constraint and CR1

- Branch office constraint: the entire 10.23.128.0/17 (32,768 addresses) is reserved and untouched. Opening a branch requires only allocating from this pool — no re-addressing of the HQ site.

- CR1 (+8 staff, no redesign): Warehouse/Logistics (VLAN 30) is sized at /26 (62 usable addresses) against a current headcount of 12. Adding 8 staff brings usage to 20/62 — well inside the existing subnet, so no VLSM change, no re-cabling, and no re-IP-addressing is required.

- General headroom: 126 further /24 networks remain free inside the HQ /17 alone, before any branch-office space is touched — this design will not need revisiting for the foreseeable life of the business.

# 5.  Initial GitHub Repository

Repository name: tshimo-agri-network-cmpg325   (suggested — public or private per module instructions)

## 5.1  Folder structure

- README.md — project overview, client summary, milestone tracker, links to each section below

- docs/01-client-requirements.md

- docs/02-physical-topology.md  (+ physical_topology_pro.png)

- docs/03-logical-topology.md  (+ logical_topology_pro.png)

- docs/04-ip-addressing-plan.md

- docs/05-change-requests.md

- packet-tracer/  — final .pkt file goes here in later milestones

- screenshots/  — connectivity and configuration evidence

- troubleshooting/  — fault isolation documentation (assigned challenge)

## 5.2  README skeleton

# Tshimo Agri Supplies — Network Design (CMPG325-2026-042)

Individual semester project for CMPG 325 Computer Networks, NWU. Client: Tshimo Agri Supplies (Potchefstroom, Agriculture). Assigned challenge: Network Troubleshooting (fault isolation scenario).

## Milestones

- Milestone 1 — Client Design Review (28 Aug 2026): requirements, topology, addressing plan — this repository

- Milestone 2 — 2 Oct 2026

- Final submission — 16 Oct 2026

## Contents

- See /docs for the full design documentation.

## 5.3  Initial commit checklist

- git init in project folder, add README.md and /docs skeleton

- Commit 1: "Add client requirements and project brief summary"

- Commit 2: "Add physical and logical topology diagrams"

- Commit 3: "Add IP addressing / VLSM plan"

- Push to GitHub and paste the repository link into the eFundi text box alongside this document